Exercise 5b. Overall vulnerability assessment using Spatial Multi Criteria Evaluation for qualitative risk assessment.

Expected time: 	3 hours

Data: 	data from subdirectory: RiskCity_exercises/exercise05b/data

Objectives: 	In this exercises you will generate a number of indicators for social vulnerability, based on different administrative units. Also indicator maps of physical vulnerability will be generated, as well as some capacity indicators. The social and physical vulnerability indicators are combined into an overall vulnerability indicator using Spatial Multi Criteria Evaluation.

# Introduction

Spatial multi criteria evaluation is a technique that assists stakeholders in decision making with respect to a particular goal (in this case a qualitative risk assessment). It is an ideal tool for transparent group decision making, using spatial criteria, which are combined and weighted with respect to the overall goal. For implementing the analysis in the RiskCity case study, the SMCE module of ILWIS was used (ITC, 2001). The input is a set of maps that are the spatial representation of the criteria, which are grouped, standardized and weighted in a criteria tree. The theoretical background for the multi-criteria evaluation is based on the Analytical Hierarchical Process (AHP) developed by Saaty (1980).

Objective   A qualitative risk assessment for defining the options of risk reduction using a multi criteria evaluation.

Risk reduction can be achieved if one of the factors of the risk equation is reduced.

Here spatial multi criteria evaluation is used to assess the overall vulnerability factor. The overall vulnerability is a function of the vulnerability and the capacity.

“Capacity is a combination of all strength and resources available within a community or organization that can reduce the level of risk, or the effect of a disaster. It may include physical, institutional, social or economic means as well as skilled personal or collective attributes such as leadership and management. Capacity may also be described as capability” (UN-ISDR, 2004)

The main goal of the SMCE is to define the overall vulnerability.

Sub-goals are:

-      to define the generic social vulnerability,

-	to define the population vulnerability for natural hazards considering a day time and night time scenarios.;,

-	to define the physical vulnerability in terms of buildings at risk for specific natural hazards,

-	and to define the capacity.

The figure below gives an overview of the goal, sub-goals and indicators used to define the sub-goals, in the Risk City exercise.

Overall vulnerability

Procedure

For implementing the semi-quantitative model, the SMCE module of ILWIS-GIS was used. The SMCE application assists and guides users when performing multi-criteria evaluation in a spatial manner (ITC, 2001). The input is a set of maps that are the spatial representation of the criteria, which are grouped, standardised and weighted in a ‘criteria tree.’ The output is one or more ‘composite index map(s),’ which indicates the realisation of the model implemented. The theoretical background for the multi-criteria evaluation is based on the Analytical Hierarchical Process (AHP) developed by Saaty (1980).

General steps in the process are:

- Definition of the problem. Structuring of the problem into a criteria tree, with several branches or groups, and a number of factors and/or constraints.

- Standardization of the factors. All factors may be in different format (nominal, ordinal, interval etc.) and should be normalized to a range of 0-1. SMCE has some very handy tools for that especially for value data, making use of different transformation graphs.

- Weighting of the factors within one group. SMCE has some very handy tools for that derived from Analytical Hierarchical Processing (AHP), such as pair wise comparison and rank ordering.

- Weighting of the groups, in order to come to an overall weight value.

- Classification of the results.

Note: it is also possible to carry out the steps independently and also to skip one or more. If you are working in a group these topics could be done by individual team members.

It is also possible to carry out the full analysis in one criteria tree (next page). However, we advise to do it in the individual components described above.

A criteria tree may contain:

Main goal:  One main goal is obligatory for any criteria tree. The main goal is also called the main root.

Constraint:  Constraints are binding criteria so no compensation is allowed. Areas in an input map (added as a constraint) that do not satisfy a constraint condition, will obtain a composite index value of 0, no matter how well these areas perform in any other criterion (factor). Constraints can only appear

directly under the main goal.

Factor:  Factors allow for compensation. Poor performance in one criterion can be compensated by good performance in another criterion. Factors may appear directly under the main goal or under a group

of factors (sub-goal), or even under a sub-sub-goal (objective). A factor can be a benefit (the higher the value, the better), or a cost (the higher the value, the worse).

Group of Factors:  A Group defines an intermediate or a partial goal. Under a Group, you can add one or more Factors and/or other Groups of Factors. Click the plus sign in front of a Group of Factors to expand the group.

Figure 3 gives an overview of the final criteria tree that can be used for this exercise. It explain the procedure and terminology used in the ITC_ILWIS SMCE tool.

One main goal is obligatory for any criteria tree. The main goal is also called the main root

a Benefit: contributes positively to the output; the more you have (the higher the values), the better it is

A Group defines an intermediate or a partial goal.

Under a Group, you can add one or more Factors and/or other Groups of Factors. Click the plus sign in front of a Group of Factors to expand the group.

The Standardization method is indicated here.

A Subgoal is directly under the main goal, it defines the main groups that together define the overall goal. Each subgoal also has a weight value.

Here are the input tables and columns that contain the data related to the factor

One subgoal can consist of one or more factors. These can be spatial or non spatial. They are all having a weight (value in front) and a standardization method ( e.g. Std: Goal)

The 4 blocks in the criteria tree refer to the main subgoals, indicated above.

# Input data

The following data are used in this exercise.

| Name | Type | Meaning |
| --- | --- | --- |
| Elements at risk |  |  |
| Mapping_units | Polygon | Building blocks of the city |
| Mapping_units | table | Table containing general statistical information on the number of buildings and people per building block |
| Wards | Polygon | Ward of the city |
| Wards | Table | Table with population information derived from census data for the wards in the city |
| Districts | Polygon | Districts of the city |
| Districts | Table |  |
| Losses for different types of hazards |  |  |
| Flood_risk_buildings Seismic_risk_buildings Technological_risk_buildings Landslide_risk_buildings | Tables | Tables with the results of the loss estimations for flooding, earthquakes, landslides and technological hazards for buildings. These are the results of the previous exercises |
| Flood_risk_population Seismic_risk_population Technological_risk_population Landslide_risk_population | Tables | Tables with the results of the loss estimations for flooding, earthquakes, landslides and technological hazards for buildings. These are the results of the previous exercises |
| Other data |  |  |
| High_res_image | Raster | High resolution image of the study area. |

The general approach: selection of the indicators for the different sub goals.

The sub goals are defined using different indicators. Based on the Risk City data set the following 4 sets of indicators were generated:

1. Social vulnerability indicators, indicated in table 1, such as:

- Percentage of young children

- Percentage of elderly people

- Population density in daytime

- Population density in nighttime

- Percentage of minority groups

- Percentage of single parent households

- Percentage of households living below poverty level.

- Literacy rate

2. Population vulnerability indicators (indicated in table 2)

- Number of people located in flood hazard zones, with different return periods, and with both a daytime and nighttime scenario

- Number of people located in landslide hazard zones, with different degree of susceptibility to landslides, and with both a daytime and nighttime scenario

- Number of people located in technological hazard zones, with different degree of susceptibility to landslides, and with both a daytime and nighttime scenario

- Number of people located in seismic hazard zones, with different intensities and return periods and with both a daytime and nighttime scenario

3. Physical vulnerability indicators (indicated in table 3)

- Number of buildings located in flood hazard zones, with different return periods

- Number of buildings located in landslide hazard zones, with different degree of susceptibility to landslides

- Number of buildings located in technological hazard zones, with different degree of susceptibility to landslides

- Number of buildings located in seismic hazard zones, with different intensities and return periods

4. Capacity indicators

- Awareness

- Open the map Mapping_units , and add the maps Wards, and Districts. Rasterize these maps; use geo reference Somewhere. - Use PixelInformation to find out the information from the attributes linked to these maps.

Table 1: Overview of available data for Social vulnerability assessment

| Map | Table | Column | Meaning |
| --- | --- | --- | --- |
| Districts | Districts | Age_under_4 | Percentage of young children, of pre-school age |
|  | Districts | Age_4_to_12 | Percentage of children, of primary school age |
|  | Districts | Age_12_18 | Percentage of teenagers, of secondary school age |
|  | Districts | Age_18_24 | Percentage of adolescents, following further education |
|  | Districts | Age_24_65 | Percentage of population in working age |
|  | Districts | Age_over_65 | Percentage of retired people. |
|  | Districts | Minor | Percentage of population coming from minority groups. |
| Wards | Wards | Nr_buildings | Number of buildings per ward |
|  | Wards | Daytime_population | Daytime population per ward |
|  | Wards | Nighttime_population | Nighttime population per ward |
|  | Wards | Unemployment | Unemployment rate per ward |
|  | Wards | Literacy_rate | Literacy rate per ward |
| Mapping units | Mapping units | Pred_landuse | Predominant landuse per mapping unit |
|  | Mapping units | PerVacant | Percent of mapping units that is vacant and could be used as shelter area, if it has the right landuse |
|  | Mapping units | Percent_single_ household | Percentage single household per mapping units |
|  | Mapping units | Poverty_level | Percentage of population in mapping unit living below poverty level |

Apart from the social vulnerability indicators, we also take into account the population vulnerability indicators, which are given in the table below.

Table 2: Overview of available data for population vulnerability assessment.

| Map | Table | Column | Meaning |
| --- | --- | --- | --- |
| Table: Mapping units Indicator: Flood risk to people | Flood_risk_population | day_pop_aff_10_year day_pop_aff_50_year | Number of people affected by a flood with a return period of 10 or 50 years, during daytime |
|  | Flood_risk_population | night_pop_aff_10_year night_pop_aff_50_year | Number of people affected by a flood with a return period of 10 or 50 years, during nighttime |
| Table: Mapping units Indicator: Landslide risk to people | Landslide_risk_population | Pop_night_high Pop_night_moderate Pop_night_low | Number of people living in the high, moderate orlow landslide susceptible zones during the nighttime |
|  | Landslide_risk_population | Pop_day_high Pop_day_moderate Pop_day_low | Number of people living in the high, moderate and low landslide susceptible zones during the daytime |
| Mapping units Indicator: Technological risk to people | Technological_risk_population | Pop_day_sc1 | Number of people being present in the area that might be affected by pool fire during the day |
|  | Technological_risk_population | Pop_night_sc1 | Number of people being present in the area that might be affected by pool fire during the night |
|  | Technological_risk_population | Pop_day_sc2 | Number of people being present in the area that might be affected by BLEVE (explosion) during the day |
|  | Technological_risk_population | Pop_night_sc2 | Number of people being present in the area that might be affected by BLEVE (explosion) during the night |
| Mapping units Indicator: Seismic risk to people | Seismic_risk_population | VI_night_pop VII_night_pop VIII_night_pop IX_night_pop | Population in buildings of buildings that collapse under VI – IX earthquakes in the night |
|  | Seismic_risk_population | VI_day_pop VII_day_pop VIII_day_pop IX_day_pop | Population in buildings of buildings that collapse under VI – IX earthquakes in the night |

The third block of indicators are the physical vulnerability indicators, which are shown in table 3.

Table 3: Overview of available data for physical vulnerability assessment

| Map | Table | Column | Meaning |
| --- | --- | --- | --- |
| Mapping units | Flood_risk_buildings | Buildings_5_year | Number of buildings affected by a flood with a return period of 5 years |
|  | Flood_risk_buildings | Buildings_10_year | Number of buildings affected by a flood with a return period of 10 years |
|  | Flood_risk_buildings | Buildings_25_year | Number of buildings affected by a flood with a return period of 25 years |
|  | Flood_risk_buildings | Buildings_50_year | Number of buildings affected by a flood with a return period of 50 years |
|  | Flood_risk_buildings | Buildings_100_year | Number of buildings affected by a flood with a return period of 100 years |
| Mapping units | Landslide_risk_buildings | Nr_buildings_high | Number of buildings located in the high susceptible zones for landslides |
|  | Landslide_risk_buildings | Nr_buildings_moderate | Number of buildings located in the moderate susceptible zones for landslides |
|  | Landslide_risk_buildings | Nr_buildings_low | Number of buildings located in the low susceptible zones for landslides |
| Mapping units | Technological_risk_buildings | Nr_buildings_sc1 | Number of buildings located in the area that might be affected by pool fire |
|  | Technological_risk_buildings | Nr_buildings_sc2 | Number of buildings located in the area that might be affected by BLEVE |
| Mapping units | Seismic_risk_buildings | VI_collapse_max | Number of buildings that are expected to collapse under a VI intensity earthquake |
|  | Seismic_risk_buildings | VII_collapse_max | Number of buildings that are expected to collapse under a VII intensity earthquake |
|  | Seismic_risk_buildings | VIII collapse_max | Number of buildings that are expected to collapse under a VIII intensity earthquake |
|  | Seismic_risk_buildings | IX_collapse_max | Number of buildings that are expected to collapse under a IX intensity earthquake |

# Part A: Creating the criteria tree for Generic Social Vulnerability Assessment.

In this step we will generate in the ILWIS Spatial Multi Criteria Evaluation (SMCE) software tool, a problem tree that will be used to calculate the  generic social vulnerability.

We assume that you have some basic knowledge on SMCE, and will not explain a lot on the background. Please consult the ILWIS help if you need more information.

In general SMCE follows a number of steps ( as mentioned before) :

- Definition of the problem. Structuring of the problem into a criteria tree, with several branches or groups, and a number of factors and/or constraints.

- Standardization of the factors. All factors may be in different format (nominal, ordinal, interval etc.) and should be normalized to a range of 0-1. SMCE has some very handy tools for that especially for value data, making use of different transformation graphs.

- Weighting of the factors within one group. SMCE has some very handy tools for that derived from Analytical Hierarchical Processing (AHP), such as pair wise comparison and rank ordering.

- Weighting of the groups, in order to come to an overall weight value.

- Classification of the results.

Below we will take you through the procedure for the generic social vulnerability assessment. Later on you can do it yourself for the other groups.

A.1. Mode: Problem definition:

Which criteria to use, and how to order them? This is often one of the most difficult parts of the procedure.

The criteria tree is composed of the following criteria:

Constraints: these criteria are used to mask out the area where the goal can not be reached. In this case, where there is no social vulnerability, because there are no people living.

Factors: those are the criteria that contribute in different way to the goal (social vulnerability score in this case). We can group these into several sub-goals or groups.

- Select Operations / Raster Operations / Spatial Multi Criteria Evaluation. Select the option Problem Analysis. An empty problem tree is opened ( or select in the Operations List) - Change the goal (right click select Edit)to: - Generic Social Vulnerability, and the name of the output map (in the right side) to Social_Vulnerability. - Click on Generic Social Vulnerability and click on the insert group icon or Right click on Generic Social Vulnerability and select Insert group. Add the groups: Age_related, Income related, Ethnicity related, Social Structure Related. - Include the various spatial factors for the individual criteria, as indicated below by right-clicking on the individual criteria and inserting the spatial factors or click the insert spatial factor icon .

You could also add a constraint, called Built_up area. This would be a Boolean column (True or False) from the Mapping_unit table, in which you indicate for each mapping unit if there are built-up areas or not. This could be done by first making such a Boolean column in the Landuse table, and then joining that with the Mapping_unit table.

QUESTION: Apart from the criteria that are given here, which other indicators do you think could be used in determining social vulnerability? Name a few examples, and indicate where you could get such data from, in your own country.

Next you will have to assign the spatial data that is relevant for each of the criteria that you have defined. These are mostly coming from tables, linked to the maps Mapping_units, Wards, and Districts. All age related data is available only at district level. Note: red areas in SMCE mean that data is still not defined.

- Double click on the red area next to Young_children. Select from the map Districts the column: Age_under_4. - Find also the relevant spatial information for the other criteria, and the result is indicated below - Save the criteria tree as Social_vulnerability..

The criteria tree should look like the example shown here to the left.

- Note: all parts indicated in red should be completed before you can make the output map.

- Adding output maps : Double click on the green area next to “ Age_related” and fill in Age_related; Press enter - Do the same for Income_related, Ethnicity_related and Social_stgructure related.

A.2. Mode Multi Criteria Analysis: Standardization of the factors.

In this case all the factors used in the social vulnerability assessment are of the “value” type, and they are

all stored as attributes in an attribute table linked to one map. Next we need to standardize these

different values, and normalize them to values ranging from 0 to 1.

Standardization and the design of value functions is a crucial part of the SMCE. Different standardization methods express different utility of input values. When standardizing, depending on the type of input map, a dialog box will appear in which you can choose the "value function" by which the map or column values are converted to values between 0 and 1. Standardization is part of the Multi Criteria Analysis mode in ILWIS-SMCE.

- In the SMCE window, change the Mode from “Problem Definition” to “Multi Criteria Analysis”. Now you can start standardization. - Double click on the red area indicating 0.00 Young_children. Now a window opens in which a graph is shown fitting the data range of values for this factor over the range of 0-1.

How to standardize?

You have to define yourself the ranges between you standardize. Consider for each factor: how much should the value be in order to consider it very vulnerable? For instance: how large should the percentage elderly people per mapping unit be to give it a 1 value (highly vulnerable). These threshold values are often defined in a group decision making process through workshops etc. Here discuss these values with your neighbors

Maximum: The input values are divided by the maximum value of the map;

Interval: Linear function with the maximum and minimum values of the map;

Goal: Linear function with specified maximum and minimum values;

Piecewise linear: Linear function with two breaking points located between the extremes

Convex: Convex function with one user defined value to re-shape the curve;

Concave: Concave function with one user defined value to re-shape the curve;

U-Shape: U-shape curve with one user defined value to stretch or shrink the curve GaussianBell-shape curve with one user defined value to stretch or shrink the curve.

You have the option to select several ways of scaling the values between 0 and 1. The figure below shows the standardization window, and the various options.

When selecting the boundaries for standardization, you always have to consider the aim of the weighting and standardization procedure (in this case social vulnerability), and how this particular variable is related to that. In this case: the higher the percentage of children in an area, the higher the vulnerability of the population. In that case you can use a simple straight line, between 0 and the maximum value. In other cases there will be a maximum value above which you will always find it high. E.g. for the estimation of the population losses, you could say that anything above 20 is high, and should be 1. In that case you select the Goal option, and you can adjust the values manually.

- Select the goal option and change the minimum X to 0 and the maximum to 20. Do the same for the variable Elderly_people. - Standardize in the same way the other variables.

After standardizing all factors, your criteria tree will look like the one below in the picture. The red bars are showing the places where still you need to indicate weights.

- To see the result of the standardization: Right click on the name Young_children and select Show standardized. A map opens that contains the standardized values. - Open PixelInformation and add the map you just created and also the map district , linked to the table District. Compare the original values to the standardized values.

A.3. Mode Multi Criteria Analysis: Determining the weights among         factors

The third step in the procedure is to define the weights between the various factors. This can be between the factors in the same group (e.g. the two factors “Young_children” and “Elderly_people” in the group “Age related”), or the weights among the groups (e.g. “Age related” versus “Income related”). There are two groups that have only one factor, and therefore the weights for these two are 1 (see above: “Minority groups”, and “Single parent households”).  For the determination of weights SMCE use 3 different methods:

Weights

- Weights are always numbers between 0 and 1.

- Weights cannot be negative.

- For the factors within a group, the sum of the weights of the factors equals 1.

- When a group only has one child, this child automatically obtains weight 1.

- Constraints are not considered during weighing.

- Direct weights (you indicate the weights directly in a table),

- Pairwise comparison (you compare the factors in pairs, and based on the consistency of your selection and relative importance, quantitative values are given to the factors), and

- Rank ordering (you indicate the relative ranking of the factors, and the software converts these in quantitative weights).

In this exercise we will work mostly with pairwise comparison.

- Right-click the red indicated factor group “Age related”, and select Weight. Select the option: Pairwise - Determine whether for the determination of social vulnerability, the percentage of young-children is more important than the percentage of elderly people, or equal, or less. Discuss this with your neighbors / group members. - Double-click in the green area next to age related and fill in age_related; Press enter. Double-click on the map name and generate the map. View the result. - Standardize in the same way the other groups e.g. “Income related” and make the intermediate maps for Income related , Etnicity related and Social structure related..

The criteria tree will then look like the one to the left.

A.4 Mode Multi Criteria Analysis : Determining the weights among groups

The fourth step in the procedure is to define the weights between among the groups (e.g. “Age related” versus “Income related”). There are four groups in this example. Also here pair wise method could be used, but you might also try out another one.

- Right-click the red indicated upper line “Generic social vulnerability ”, and select Weight. Select the option: Pairwise - Determine for each combination the relative importance (see below). Discuss this with your neighbors / group members. .

The resulting criteria tree might look like the one below (but the weights could be different, depending on the importance you gave to the different groups of factors.)

Now all the parameters are given and it is time to calculate the output map.

- Right-click the map icon “Social_vulnerability”, and select Generate selected item. - Display the result map. Use PixelInfo to compare the resulting map with the input maps. You can adjust the standardization, and weights if you would like to make adjustments.,

Question

What can you conclude from the pattern of social vulnerability?

# Part B. Creating the criteria tree for Population Vulnerability Assessment ( using hazard specific indicators)

In this part you will generate the maps required for the population vulnerability indicators using spatial multi-criteria analysis. The population that might be affected by earthquakes, landslides, flooding and technological disasters during a daytime and nighttime scenario, will be combined into one population vulnerability.

B.1 Preparation of input maps

In this step we will generate the maps required for the spatial multi-criteria analysis. In the SMCE software each table containing columns that are used as indicators should be linked to a raster map. As most of the attribute tables with the results of population and buildings losses are linked to the mapping_units map, we need to copy this map several times, so that each table has its own map.

- Rasterize the polygon maps Mapping_units, Wards, and Disticts using the Georeference Somewhere if this hasn’t been done yet. - Select the map mapping_units and select the Edit / Copy Object to and select New Name. Name the file: Flood_risk_buildings. - Change the properties of the raster map Flood_risk_buildings, and make sure it is linked to the table Flood_risk_buildings. - Do the same for all the files in the table listed below, and give them the names as indicated.

## Table 4: Copy the raster map Mapping_units to these names, and link each one of them to the table with the same name

| Table names. |  |
| --- | --- |
| Flood_risk_buildings | Seismic_risk_buildings |
| Flood_risk_population | Seismic_risk_population |
| Landslide_risk_buildings | Technological_risk_buildings |
| Landslide_risk_population | Technological_risk_population |

B.2 Generating the criteria tree for Population vulnerability

Once the input maps have been generated, you can start with the generation of the criteria tree and the multi criteria analysis. As the procedure was already explained in the previous section, we will not repeat it here again.

- Create a new criteria tree: Population_Vulnerability, and the file name also the same. - Add groups of the individual groups of factors: Earthquake_losses, Landslide_losses, Flood_losses, Technological_losses. - Include for each hazard type, two subgroups: Nighttime losses, and Daytime losses. - Enter the most relevant scenarios for each hazard type. For example, for earthquakes, only adding the IX scenario would be enough. You can compare it with the figure below, but you don’t have to do it exactly the same

B.3 Standardizing and weighting

Once the criteria tree is made, you can define the related attributes and start the standardization. As the procedure was already explained in the previous section, we will not repeat it here again.

- Choose the relevant attributes from the “Population risk” columns linked to the maps of the Mapping_units for earthquakes, landslides, floods and technological hazards. - Standardize all columns, using the same “Goal” function with for example the value 100 as the one reaching 1. - Use the weighting of the daytime-nighttime losses, using a same weight of 0.5 - Use the pairwise method for the hazard and state which hazard you find more important than others. - Generate the output map Population_vulnerability, and critically evaluate the result. If needed, adjust the criteria tree. An example of a possible criteria tree is given below. - Do you think that the parameters taken into account are good indicators for the evaluation of the vulnerability? Do you have other ideas?

# Part C.    Creating the criteria tree for Physical Vulnerability Assessment.

In this part you will generate the maps required for the hazard specific physical vulnerability indicators using spatial multi-criteria analysis. The procedure for estimating the number of buildings that might be affected by earthquakes, landslides, flooding and technological disasters will be further explained in the exercises of session 6. Here we will combine them into one physical vulnerability index.

- Create a new criteria tree: Physical_Vulnerability, and name the output file name also the same. - Add groups of the individual groups of factors: Earthquake_losses, Landslide_losses, Flood_losses, Technological_losses. - Include for each hazard type, all the calculated scenarios for each hazard type. For example, for earthquakes, add scenarios VI, VII, VIII and IX intensity. - Choose the relevant attributes from the “Building risk” columns linked to the maps of the mapping units for earthquakes, landslides, floods and technological. - Standardize all columns, using the same “Goal” function with for example the value 25 as the one reaching 1. - Use the pairwise method for the scenarios within each hazard category - Also use the pairwise method for comparing the various hazards and state which hazard you find more important than others - Generate the output map Physical_vulnerability, and critically evaluate the result. If needed, adjust the criteria tree. An example of a possible criteria tree is given below.

# Part D. Creating the criteria tree for Capacity Assessment.

The overall vulnerability is a function of vulnerability and capacity. Capacity expresses the positive managerial and operational resources and procedures for reducing risk factors. These actually help to reduce the vulnerability.

In our case study we are using only one capacity indicators: awareness level, expressed by the literacy rate. The capacity indicator should work opposite to the other vulnerability indicator.

- Create a new criteria tree: Capacity, and the file name also the same. - Add the group: Disaster_Awareness. - Under this group, include one spatial factor: Literacy_rate Select the column Literacy_rate from the table Wards. - Standardize the factor, keeping in mind that high values of literacy rate results in high values of the capacity index. - Generate the output map Capacity, and critically evaluate the result.

# Part E. Combing vulnerability and capacity indicators

The overall vulnerability  is made by combining the four “indicators” that we have calculated thus far in:

- Social_Vulnerability Assessment (Part A)

- Population_Vulnerability (Part B)

- Physical_Vulnerability (Part C)

- Capacity Assessment (Part D)

- Create a new criteria tree: Overall_vulnerability and the file name also the same. - Add the four spatial factors: Social_vulnerability, Population_vulnerability, Physical_vulnerability and Capacity. - Link them to the four maps that were made in Part A, B, C and D. - Standardize the four factors, and use the pairwise method for the determination of the weights. - The capacity will be made a cost, since it is lowering the vulnerability. - Generate the output map Overall _vulnerability.

- Classify the output map in three classes and critically evaluate the result. (Create a histogram from the Overall_vulnerability and select 3 classes).

WHICH AREAS HAVE THE HIGHEST VULNERABILITY?